A Brief Tutorial for Xilinx Vivado

VE270 TA Group

Updated 2020.05.11

NOTE: The following tutorial is based on Vivado version 2019.2.

- Introduction

Xilinx Vivado is a newer version of Xilinx software. In this semester, it’s mainly used for burn the program to the board as well as Verilog programming.

- Installation

- Download & Sign Up

Download Vivado Design Suite HLx Edition from Xilinx Website (xilinx.com). You need to sign up at xilinx.com.

Based on your edition and network, you can choose to download the “Self Extracting Web Installer” or “All OS Installer Single-File”.

The former installer will download the main program by itself and the latter one has contained all necessary files and you needn’t download anything more.

Here, we use the former one to continue demo.

- Execute the installer and you need to sign in. Choose Vivado WebPack which is free. Default tools are enough for our labs.

Tips:

- If you want to modify the installed content (for example like SDK etc.), you could manage them through Xilinx Information Center – Add Tools/Devices.

- If you want to modify the installation directory, make sure it does not contain any spacing inside the directory. Namely “E:\Xilinx Vivado” is illegal and the correct way is shown in the picture below.

- When downloading, there may be warnings as the figure below,

If the installer is still running and the progress bar is progressing, please just ignore it or click “Retry” several minutes later, it will not affect the downloading process.

- If you find the downloading speed to be slow, connect to SJTU VPN may help.

- Start early. The installation of Vivado does take a while.

- Create Project

- Open Vivado to open the software.

- Click Create Project in the left side panel.

- Follow the navigator to choose project file location and project name:

Project Type:

Select RTL Project option in the Project Type form, and click Next.

you could check “Do not specify sources at this time” box if you want to add source file later.

Add Source (Can Skip):

Optional:

Using the drop-down buttons, select Verilog as the Target Language and Simulator Language in the Add Sources form.

Add Constrain (Can Skip):

Choose Default Part:

There are two ways to choose parts, select by Parts or by Boards:

- Select by Parts:

Search and Choose parts:

XC7A35TCPG236-1 for Basys3, or XC7A100TCSG324-1 for Nexys4 DDR.

- Select by Boards:

- According to the instruction from Digilent (digilentinc.com) website, download Digilent Vivado boards file from github.com.

- Copy folders in “new/board_files” into “vivado install location/Vivado/version/data/boards/board_files”. For detail, please refer to the instruction on Digilent website.

- You will find board file for Basys3 or Nexys4 DDR:

Finish Creating Project:

Then a new project is created.

- Design and Simulation

- IP Integrator Design

Please refer to Vivado Design Hub – Using IP Integrator and Vivado Design Suite Tutrial: Designing IP Subsystems Using IP Integrator.

- Verilog Programmatically Design

Find source windows and click “+” to add source.

Choose “Add or create design sources”.

Click Create Files

Choose File type to be Verilog, enter File name.

Click “OK” and “Finish”, then a “Define Module” window will pop up. You can skip this step, or you can also name your module ports here.

Then you can code in your Verilog file to implement your design.

- Create Simulation

Find source windows and click “+” to add source.

Choose “Add or create design sources”.

Click “Create File”, Name file and choose location.

Click OK and Finish.

Open the simulation Verilog file in Source Hierarchy to write your simulation.

Choose corresponding simulation file and run simulation by clicking “Run Simulation” in the left Flow Navigator bar and get the result.

- Implementation on FPGA Board

- Generate .bit file

Run Synthesis in Flow Navigator with default setting.

Click “Run Implementation” in Flow Navigator.

Choose “I/O Planning” of panel view in upper right drop down menu.

Select “I/O Port” tab in the bottom window. You can open this window in “Window->I/O Port”:

Then you can assign input and output pin number according to board Reference Manual to “Package Pin” column.

Save the change and rerun Synthesis and Implementation.

Connect your board to your computer. And click Generate Bitstream. Allow device auto connect.